## Pump Not Reaching Cut-Off Pressure?

### Customer Troubleshooting Checklist

Provided by Buffalo Farm Equipment

Before you call out a technician, please work through this list to help identify the problem:

### ✅ 1. Check Your Water Source

- Is there water in the tank, bore, dam, or other supply?

- Are all valves open from the water source to the pump?

- Is your pump primed (for surface pumps)?

- Are there any visible air leaks or cracks in the suction pipe?

### ✅ 2. Look for Leaks or Flow Issues

- Are any taps, sprinklers, or valves left open after the pump?

- Do you see any leaks in the pipeline or fittings?

- Can the pump build pressure if all outlets are closed?

### ✅ 3. Check the Pump and System

- Is the pump making any strange noises (grinding, rattling, humming)?

- Is the motor running but not building pressure?

- Is the impeller blocked or worn out?

### ✅ 4. Inspect the Pressure Switch or Controller

- If you have a pressure switch:

  - Are the settings correct (not set too high)?

  - Is it clicking or operating normally?

- If you use a pressure controller:

  - Are any error lights flashing?

### ✅ 5. Check the Pressure Tank (if fitted)

- Is the tank holding air pressure?

- Can you hear water sloshing when you tap it (might mean failed bladder)?

- Has the tank been serviced recently?

### ✅ 6. Power Supply

- Has a fuse, circuit breaker, or RCD tripped?

- Are you running on a generator? (It must be the right size and voltage)

- Is the plug/socket connection tight and dry?

### Still Not Working?

Write down your pump model and what you’ve checked, and call:

📞 Buffalo Farm Equipment – (03) 5752 1234
Or email: admin@buffalofarmequipment.com.au

We're here to help you get back up and running.