# How to Get Your Fire Pump Ready for Winter Storage

If your fire pump will not be used over winter, taking a few simple steps now can prevent expensive repairs and ensure it starts when you need it.

## 1. Test Run the Pump

Run the pump with water connected and flowing for 5–10 minutes to ensure everything is operating correctly.

Never run the pump dry. Running without water can damage the mechanical seal and pump components.

## 2. Turn Off the Fuel

Close the fuel tap and allow the engine to run until it stops. This removes fuel from the carburettor and helps prevent fuel-related starting problems.

## 3. Drain the Pump

Drain all water from the pump casing, suction hose, and delivery hose. This helps prevent corrosion and damage during storage.

## 4. Check the Engine Oil

Check the oil level and condition. If the oil is dirty, change it before storing the pump.

## 5. Clean the Pump

Remove dirt, grass, leaves, and debris from the engine and pump body. Ensure cooling fins and air intake areas are clean.

## 6. Inspect Hoses and Fittings

Check hoses, clamps, strainers, and fittings for wear, cracks, or leaks.

## 7. Battery Maintenance (Electric Start Models)

Disconnect the battery and store it in a dry place. Recharge periodically during storage.

## 8. Store Under Cover

Keep the pump in a dry, sheltered location protected from weather and direct sunlight.

## 9. Protect Against Vermin

Mice and insects can damage wiring, air filters, and engine components. Check the pump before putting it back into service.

## 10. Before Fire Season

Before relying on the pump:

- Check oil level

- Add fresh fuel if required

- Prime the pump

- Run the pump with water

- Check pressure and flow

- Inspect all hoses and fittings

### Buffalo Farm Equipment Tip

Test your fire pump before every fire season. A quick test now is much better than finding a problem during an emergency.